# HANDRAILS – SECTION 1003.3.3.6 1997 UBC

1003.3.3.6  Handrails.  Stairways shall have handrails on each side, and every stairway required to be more than 88 inches (2235 mm) in width shall be provided with not less than one intermediate handrail for each 88 inches (2235 mm) of required width.  Intermediate handrails shall be spaced approximately equally across with the entire width of the stairway.

EXCEPTIONS:

- Stairways less then 44 inches (1118 mm) in width or stairways serving one individual dwelling unit in Group R. Division 1 or 3 Occupancies or a Group R, Division 3 congregate residence may have one handrail.

- Private stairways 30 inches (762 mm) or less in height may have handrails on one side only.

- Stairways having less than four risers and serving one individual dwelling unit in Group R, Division 1 or 3, or a Group R, Division 3 congregate residence or serving Group U Occupancies need not have handrails.

The top of handrails and handrail extensions shall be placed not less than 34 inches (864 mm) or more than 38 inches (965 mm) above the nosing of treads and landings.  Handrails shall be continuous the full length of the stairs and at least one handrail shall extend in the direction of the stair run no less than 12 inches (305 mm) beyond the top riser nor less than 12 inches (305 mm) beyond the bottom riser.  Ends shall be returned or shall terminate in newel posts or safety terminals.

EXCEPTIONS:

- Private stairways do not require handrail extensions.

- Handrails may have starting or volute newels within the first tread on stairways in Group R, Division 3 Occupancies and within individual dwelling units of Group R, Division 1 Occupancies.

The handgrip portion of handrails shall not be less than 1 ¼ inches (32 mm) nor more than 2 inches (51 mm) in cross-sectional dimension or the shape shall provide an equivalent gripping surface.  The handgrip portion of handrails shall have a smooth surface with no sharp corners.  Handrails projecting from a wall shall have a space of not less than 1 ½ inches (38 mm) between the wall and the handrail.